Computer Networks Project

Done by:

Fatema Al Zahraa Sherif

Norhan Hazem Mohammed

Yasmen Yasser

Nour Mohey

TABLES:

Part 1: Design and Implement a VLSM Addressing Scheme.

//reordering

10.1.0.0/12

| VLAN 10 | 61 | 62 | 10.1.0.0/26 | 1 | 62 | 63 |
| --- | --- | --- | --- | --- | --- | --- |
| VLAN 70 | 31 | 62 | 10.1.0.64/26 | 65 | 126 | 127 |
| VLAN 20 | 30 | 30 | 10.1.0.128/27 | 129 | 158 | 159 |
| VLAN 60 | 29 | 30 | 10.1.0.160/27 | 161 | 190 | 191 |
| VLAN 80 | 25 | 30 | 10.1.0.192/27 | 193 | 222 | 223 |
| VLAN 40 | 20 | 30 | 10.1.0.224/27 | 225 | 254 | 255 |
| VLAN 50 | 15 | 30 | 10.1.1.0/27 | 1 | 30 | 31 |
| VLAN 25 | 12 | 14 | 10.1.1.32/28 | 33 | 46 | 47 |
| From SW-S to MIU-MIU-GW | 7 | 14 | 10.1.1.48/28 | 49 | 62 | 63 |
| MAIN-MLS to MIU-GW | 2 | 2 | 10.1.1.64/30 | 65 | 66 | 67 |
| From N-MLS to MIU-MIU-GW | 2 | 2 | 10.1.1.68/30 | 69 | 70 | 71 |
| From S-MLS to MIU-MIU-GW | 2 | 2 | 10.1.1.72/30 | 73 | 74 | 75 |
| From R-MLS to MIU-MIU-GW | 2 | 2 | 10.1.1.76/30 | 77 | 78 | 79 |

Subnet Table

| 1.Subnet Description | VLAN ID | VLAN Name | Required Number of Hosts | Network Address/CIDR | First Usable Host Address | Last Usable Host Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Main | VLAN 10 |  | 61 | 10.1.0.0/26 | 1 | 62 | 63 |
|  | VLAN 20 |  | 30 | 10.1.0.128/27 | 129 | 158 | 159 |
| S | VLAN 25 |  | 12 | 10.1.1.32/28 | 33 | 46 | 47 |
|  | VLAN 40 |  | 20 | 10.1.0.224/27 | 225 | 254 | 255 |
| N | VLAN 50 |  | 15 | 10.1.1.0/27 | 1 | 30 | 31 |
|  | VLAN 60 |  | 29 | 10.1.0.160/27 | 161 | 190 | 191 |
| R | VLAN 70 |  | 31 | 10.1.0.64/26 | 65 | 126 | 127 |
|  | VLAN 80 |  | 25 | 10.1.0.192/27 | 193 | 222 | 223 |
| From Main-MLS to MIU-MIU-GW | - |  | 2 | 10.1.1.64/30 | 65 | 66 | 67 |
| From N-MLS to MIU-MIU-GW | - |  | 2 | 10.1.1.68/30 | 69 | 70 | 71 |
| From S-MLS to MIU-MIU-GW | - |  | 2 | 10.1.1.72/30 | 73 | 74 | 75 |
| From R-MLS to MIU-MIU-GW | - |  | 2 | 10.1.1.76/30 | 77 | 78 | 79 |
| From SW-S to MIU-MIU-GW | - |  | 7 | 10.1.1.48/28 | 49 | 62 | 63 |

Addressing Table

| Device | Interface | Address | Subnet Mask | Default Gateway | VLAN ID |
| --- | --- | --- | --- | --- | --- |
| Main-MLS | G1/1/1 | 10.1.1.65/30 | 255.255.255.252 | - | - |
|  | VLAN 10 | 10.1.0.1/26 | 255.255.255.192 | N/A |  |
|  | VLAN 20 | 10.1.0.129/27 | 255.255.255.224 |  |  |
| S-MLS | G1/1/1 | 10.1.1.73/30 | 255.255.255.252 |  |  |
|  | VLAN 25 | 10.1.1.33/28 | 255.255.255.240 |  |  |
|  | VLAN 40 | 10.1.0.225/27 | 255.255.255.224 |  |  |
| N-MLS | G1/1/1 | 10.1.1.69/30 | 255.255.255.252 |  |  |
|  | VLAN 50 | 10.1.1.1/27 | 255.255.255.224 |  |  |
|  | VLAN 60 | 10.1.0.161/27 | 255.255.255.224 |  |  |
| R-MLS | G1/0/5 | 10.1.1.77/30 | 255.255.255.252 |  |  |
|  | VLAN 70 | 10.1.0.65/26 | 255.255.255.192 |  |  |
|  | VLAN 80 | 10.1.0.193/27 | 255.255.255.224 |  |  |
| MIU-MIU-GW | Gig0/0 | 209.165.200.226/30 | 255.255.255.252 |  |  |
|  | Gig0/1 | 10.1.1.78/30 | 255.255.255.252 |  |  |
|  | Gig0/0/0 | 10.1.1.49/30 | 255.255.255.252 |  |  |
|  | Gig0/1/0 | 10.1.1.66/30 | 255.255.255.252 |  |  |
|  | Gig0/2/0 | 10.1.1.74/30 | 255.255.255.252 |  |  |
|  | Gig0/3/0 | 10.1.1.70/30 | 255.255.255.252 |  |  |
| ISP | Gig 0/0 | 209.165.200.225 | 255.255.255.240 |  |  |
|  | Gig 0/1 | 64.100.1.1 | 255.255.255.224 |  |  |
|  | Gig 0/2 | 64.100.2.1 | 255.255.255.224 |  |  |
| Branch-GW | Gig 0/0.2 | 192.168.2.1 | 255.255.255.0 |  |  |
|  | Gig 0/0.3 | 192.168.3.1 | 255.255.255.0 |  |  |
|  | Gig 0/1 | 64.100.1.2 | 255.255.255.224 |  |  |
| Wireless Home Router | G0/1 | 64.100.2.2 | 255.255.255.224 |  |  |
|  | Router IP | 192.168.10.1 | 255.255.255.128 |  |  |

Hosts Address

| PC-number | VLAN ID | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| MIU |  |  |  |  |
| PC-1 | 10 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-2 | 10 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-3 | 25 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-4 | 25 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-5 | 25 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-6 | 40 | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-7 | 50 | 10.1.1.30/27 | 255.255.255.224 | 10.1.1.1 |
| PC-8 | 60 | 10.1.0.190/27 | 255.255.255.224 | 10.1.0.161 |
| PC-9 | 70 | 10.1.0.126/26 | 255.255.255.192 | 10.1.0.65 |
| PC-10 | 80 | 10.1.0.222/27 | 255.255.255.224 | 10.1.0.193 |
| MIU_ Branch 1 |  |  |  |  |
| PC-11 |  | From DHCP Server | From DHCP Server | From DHCP Server |
| PC-12 |  | From DHCP Server | From DHCP Server | From DHCP Server |
| Wireless Home Network |  |  |  |  |
| Laptop |  |  |  |  |
| Tablet |  |  |  |  |
| Smartphone |  |  |  |  |
| Servers |  |  |  |  |
| DHCP Server | 10.1.1.62 | 255.255.255.240 | 10.1.1.49 | 10.1.1.62 |
| Email Server | 10.1.1.61 | 255.255.255.240 | 10.1.1.49 | 10.1.1.61 |
| Web server | 10.1.0.100 | 255.255.255.192 | 10.1.0.1 | 10.1.0.100 |
| DNS server | 10.1.1.58 | 255.255.255.240 | 10.1.1.49 | 10.1.1.58 |
| NTP and Syslog server | 10.1.1.59 | 255.255.255.240 | 10.1.1.49 | 10.1.1.59 |

Part2: Build the Network and Configure Basic Device Settings and Interface Addressing

Step2-a:

no ip domain-lookup

hostname MIU-GW

security passwords min-length 10

enable secret CSC1234567

line console 0

password MIU1234567

login

exit

service password-encryption

banner motd ^ Authorized Access Only! ^

step2-b:

interface gigabitEthernet 0/0

ip address 10.1.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 10.1.2.1 255.255.255.0

no shutdown

exit

step2-c:

ip domain-name miu.local

username admin privilege 15 secret MIUadmin123

crypto key generate rsa

1024

line vty 0 4

transport input ssh

login local

exit

saving: copu running-config startup-config

Part 3: Configure Network Infrastructure Settings (VLANs, Trunking, Inter-VLAN Routing EtherChannel).

Step1:

enable

configure terminal

vlan 10

name Staff

exit

vlan 20

name Students

exit

step 2:

interface fastethernet 0/1

switchport mode access

switchport access vlan 10

exit

interface fastethernet 0/2

switchport mode access

switchport access vlan 20

exit

show vlan brief

step 3:

MIU-GW Interfaces Pinging:

part 6:

- Dynamic NAT with PAT and Static NAT setup

- Web server configuration and troubleshooting

- End-to-end connectivity testing (ping/traceroute)